Juniper Networks ASV

Chartroutes

# Document Revision

1.5

8/24/2017

Michael McCoy

Juniper Networks, ASV

Table of Contents

Document Revision	1

Introduction	3

Initial Setup and Installation	3

Location of the files:	4

The topology 'Database' File	4

Enabling NETCONF and SSH on the devices	4

Example Usage	4

Introduction to the command line	5

Specifying the devices (-H):	5

Specifying the device login (-l):	6

Specifying the Chart Duration (-d):	6

Specifying the Output File Name (-f):	6

Specifying the protocol to chart (-p):	7

Specifying the routing tables to chart (-t):	7

Calculating convergence time (-s and -e):	9

# Introduction

Chartroutes.py is a Python application that uses the PyEz library to establish connections to Juniper routers and create Excel spreadsheet graphs of active and total route counts for a specified protocol based on the output of 'show route summary'. While not intended to run indefinitely, like an MRTG or other SNMP tools, it can be very useful for fixed duration testing (i.e. convergence testing).

The application allows the user to specify a single protocol to be graphed, from multiple routing tables. For example, BGP routes from inet.0 and inet6.0

Why save the data and create a graph in a spreadsheet? Because if you have the raw data set, you can easily manipulate it for reporting. Reduce the number of data points, or data sets, etc... If it's all in a spreadsheet, it's easy to regenerate the graphs.

The Chartroutes tool can also be used to calculate convergence times for a protocol in multiple tables (i.e. BGP in inet.0 and inet6.0) if starting and ending route counts are specified. Refer to the example at the end of this document

# Initial Setup and Installation

If you are installing Chartroutes on a new system, Python (all testing has been done with Python v2.7.12) , and PyEz should already be installed. If not, the following links may be helpful:

https://github.com/Juniper/py-junos-eznc

https://www.python.org/download/releases/2.7/

Additionally, the application makes use of a Python library for creating Microsoft Excel Spreadsheets, named xlsxwriter. Instructions for installing this library can be found here:

http://xlsxwriter.readthedocs.io/getting_started.html

## Location of the files:

It's not important where the Chartroutes.py file runs... just creatr a folder named Chartroutes and place it there.

## The topology 'Database' File

One of the main features of Chartroutes is that it allows the user to run the application simultaneously against multiple devices based on a regex of the device hostnames. In order to accomplish this, an Excel file of the network devices must be created, and the hosts added to the /etc/hosts file on the system on which this is running (or the hosts should be resolved by DNS). While this may sound like a bit of effort, it is trivial -- just follow the example .xlsx file provided. It is certainly easier than dealing with LRM.

Or, if you don't want to bother with any database, you can run the tool against one or more systems by IP address.

Only the Only the hostname(s) or address(es) are required fields, but there are additional tools that use the other fields in this file. It would also be easy for example to add functionality to Chartroutes that permits the user to select devices by model, or rack location instead of by hostname. This file can be stored anywhere, but the default is the Chartroutes directory. Update the path in Chartroutes.py if you prefer to store this somewhere else.

## Enabling NETCONF and SSH on the devices

Like all of the PyEz based tools, a NETCONF / SSH connection to the routers must be established for the RPC calls. On the routers, NETCONF / SSH is enabled as follows:

set system services ssh protocol-version v2

set system services netconf ssh

# Example Usage

## Introduction to the command line

Chartroutes is command line driven with a built-in help (-h) function that will list all of the supported options:

[csim@tatooine Chartroutes]$ python ./Chartroutes.py -h

Usage: chartroutes.py [-h] -H H -d D [-l L] [-t T] [-p P] [-f F] [-x X]

Script to collect and graph total/active routes for a given protocol on a

device.

optional arguments:

-h, --help  show this help message and exit

-H H        Hostname(s) of the systems to poll (supports regex)

-A A        IP address(es) of the system to poll (comma separated)

-d D        Time to poll in seconds

-l L        login:password for the system

-t T        Routing tables to poll (default: inet.0)

-p P        Protocol to poll (default: BGP)

-f F        Output file name (epoch_time.xlsx will be appended)

-x X        Enable debug to dump collected data

-u U        Use Epoch Time for graphing

-s S        Starting total route value for convergence calculation

-e E        Ending total route value for convergence calculation

## Specifying the devices (-H):

Devices on which to run the script against are specified using -H '{regex}'

[csim@tatooine Chartroutes]$ python ./Chartroutes.py -H {regex}

In this example, we have hostnames in the .xlsx file that include westford-cor-lf1, -lf2 thru westford-cor-lf8. Let's say that we only want to run this on -lf3 thru -lf6, that's accomplished as follows:

csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[3-6]'

## Specifying the device login (-l):

Login credentials are required for the SSH session to the routers and are specified using -l username:password

csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[3-6]' -l username:password

**Note that in the Chartroutes code, there is a place to specify a default username and password if one is not supplied on the command line. I recommend doing this for your individual lab setups, so that you don't have to bother specifying this option every time.

## Specifying the Chart Duration (-d):

Chartroutes will run the top command and collect the data on the specified routers (-H) for as many seconds as the configured duration (-d). However, the user can stop the collection at any time by pressing enter.

csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[3-6]' -d 60

## Specifying the Output File Name (-f):

The data collected from the routers will be saved to the ./data directory using the file name that's specified along with an epoch timestamp. If no name is specified, a default name is used

[csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[1-2]' -f BGP_CONVERGENCE_TEST -d 30

...snip...

[csim@tatooine Chartroutes]$ cd data

[csim@tatooine data]$ ls -l | grep 'Feb 10'

-rw-rw-r--. 1 csim csim   15086 Feb 10 14:30 BGP_CONVERGENCE_TEST_1486754991.xlsx

-rw-rw-r--. 1 csim csim   32696 Feb 10 14:30 westford-cor-lf1_1486754993

-rw-rw-r--. 1 csim csim   32691 Feb 10 14:30 westford-cor-lf2_1486754993

## Specifying the protocol to chart (-p):

Currently, the application allows only a single protocol to be graphed, and that's specified using the -p option. In this example, the protocol is BGP

[csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[1-2]' -f BGP_CONVERGENCE_TEST -d 30 -p BGP

## Specifying the routing tables to chart (-t):

The application allows the specified protocol to be graphed from multiple routing tables. In the example, inet.0 and inet6.0

[csim@tatooine Chartroutes]$ python ./Chartroutes.py -H 'westford-cor-lf[1-2]' -f BGP_CONVERGENCE_TEST -d 30 -p BGP -t inet0,inet6.0

The data collected from the routers will be saved to the './data' directory using the file name that's specified along with an epoch timestamp. If no name is specified, a default name is used

**Note for each execution of the application, there will be one .xlsx file with the combined data and graphs for all routers specified using the -H option (a separate sheet is created for each device)

With only a few command line options, there aren't many examples required. The following is a sample run taken on a system during convergence testing. In this example BGP routes are graphed from inet.0 and inet6.0

[csim@tatooine chartroutes]$ python ./chartroutes.py -H westford-cor-lf1 -d 900 -t inet.0,inet6.0 -p BGP -f ROUTES_IBGP_CONVERGENCE_T1

Gathering BGP route summary statistics in table(s) inet.0,inet6.0 on roswell-cor1-lf1 for 900 seconds

Starting data collection at Thu Feb 16 17:15:35 2017

Press ENTER any time to stop data collection.

Ending data collection at Thu Feb 16 17:30:40 2017

Creating file home/csim/public_html/ftp/scripts/python/chartroutes/data/ROUTES_IBGP_CONVERGENCE_T1_1487283335.xlsx with the collected data

Done.

Below is a generated spreadsheet and graph:

| inet6.0 BGP Total Routes | inet6.0 BGP Active Routes | Elapsed Time | inet.0 BGP Total Routes | inet.0 BGP Active Routes |
| --- | --- | --- | --- | --- |
| 2360 | 2080 | 345.94 | 4067301 | 704851 |
| 6840 | 3989 | 347.16 | 4073301 | 704851 |
| 11320 | 8221 | 348.39 | 4080301 | 704851 |
| 15800 | 9920 | 349.51 | 4086701 | 704851 |
| 22520 | 10169 | 350.64 | 4093501 | 704851 |
| 27280 | 10169 | 351.92 | 4099901 | 704851 |
| 31760 | 10418 | 353.09 | 4109701 | 704851 |

...

...

## Calculating convergence time (-s and -e):

The tool can also be used to calculate convergence time for a protocol in multiple tables. For example, you may want calculate the BGP RIB-IN convergence on a system that is learning both inet.0 and inet6.0 prefixes.

In the following example BGP routes in tables inet.0 and inet6.0 are exepcted to increase from 1145677 to 1235602 (inet.0) and from 430800 to 431600 (inet6.0). The script will find the starting and ending values in each table, along with the time stamp for those samples and calculate a rate:

[csim@tatooine chartroutes]$ python ./chartroutes.py -H roswell-tra1-lf4 -t inet.0,inet6.0 -d 600 -s 	1145677,430800 -e 1235602,431600 -f TEST1

Gathering BGP route summary statistics in table(s) inet.0,inet6.0 on roswell-tra1-lf4 for 600 seconds

Starting data collection at Wed Aug 23 12:15:23 2017

Press ENTER any time to stop data collection.

Data collection stopped by user after 40.6094779968 seconds

Ending data collection at Wed Aug 23 12:16:05 2017

Calculating convergence time for BGP

Convergence time(s) for roswell-tra1-lf4

Convergence time based on the total route count increasing from 1145677 to 1235602 in table inet.0

Convergence starting time = 25.49 seconds

Convergence ending time = 29.91 seconds

Convergence time (elapsed) = 4.42 seconds

Convergence Rate = 20345.02 routes per second

Convergence time based on the total route count increasing from 430800 to 431600 in table inet6.0

Convergence starting time = 31.12 seconds

Convergence ending time = 32.54 seconds

Convergence time (elapsed) = 1.42 seconds

Convergence Rate = 563.38 routes per second

Creating file /home/csim/public_html/ftp2/scripts/python/chartroutes/data/TEST1_1503504923.xlsx with the collected      	data

Done.